Speed: potentiometer speed

Length: bag length

Thing: film length

Quantity: the yield

Stick: automatical connect film          right next: next time right connect the film        disable: not use this

Sync(HD5120): Synchronization Adjustment of Cutter and Film

Mark phase(HD5130): Adjust the position of the cut point relative to the color mark

Thing phase(HD5134): adjust material put in position

Ppr: feed film pulse number per circle, the value of DS3 servo P0-11

Gear: reduction ratio of feed film, the motor rotating circles when the film roller rotating one circle

Lpr: feed film roller Perimeter

Kp: Responsiveness, the Responsiveness of servo acceleration deceleration

Feed rate: Error Compensation Rate, Speed of Compensation for Operational Deviation

Total rate: Whole machine compensation, Adjustment coefficient of color mark offset at high speed

Ppr: cutter pulse number per circle, the value of DS3 servo P0-11

Gear: cutter reduction ratio, motor rotating circles when cutter rotating one circle

Lpr: cutter roller Perimeter

Kp: Responsiveness, the Responsiveness of servo acceleration deceleration

Feedback pulse(HD1614): pulse number per circle feedback (4-time frequency)

Feed rate: Error Compensation Rate, Speed of Compensation for Operational Deviation

Num of cut(HD1634): the quantity of cutter

Height(HD1620): the Thickness of Cutter

Agl offset(HD1616): origin offset, The angle formed when the actual origin is relative to the cutting position

Ppr: feeding material pulse number per circle, the value of DS3 servo P0-11

Gear: feeding material reduction ratio, motor rotating circles when feeding material running one dial ruler

Lpr: the space of dial ruler

Kp: Responsiveness, the Responsiveness of servo acceleration deceleration

Feedback pulse: pulse number per circle feedback (4-time frequency)

Feed rate: Error Compensation Rate, Speed of Compensation for Operational Deviation

Num of Thg(HD2134): moving pass the dial ruler quantity after touching the origin once

Agl offset(HD5144): origin offset, The angle formed when the actual origin is relative to the cutting position

Max spd (bag/minute): whole machine max speed

Min spd(bag/minute): whole machine min speed

Stick spd(bag/minute): connect film speed

Acc time(second): the time accelerating to the max speed

Dec time(second):the time decelerating to the min speed

E-stop time(second): urgent stop time, after pressing the urgent stop button, the time of the whole machine decelerating to stop machine speed

Total rate: whole machine compensation

Stop spd(bag/minute): stop machine speed (HD5008)

Revise angle: torque anti-cutting angle

Mark missing: color mark missing alarm times (HD5180)

Stop angle: cutter angle when machine stop(HD5340)

Relay time(ms): relay ON time, 30ms in generally (HD5360)

Stick length: connect film distance (not useful) (HD5160)

Stick to thing: Distance from colour mark to moulder (HD5162)

Thing to cut: distance from moulder to cutter center (HD5164)

Stick time: connect film time (not useful) (HD5600)

Break check: film break checking

Func stick: connect film

Logic empty: anti-empty bag

Advance torque: torque anti-cutting

Advance mark: Photoelectricity anti-cutting

Empty mark len: the distance from anti-empty bag photoelectricity to moulder (HD5190)

Org offset: origin offset angle

Ignor num: judge whether to use anti-empty bag stop (generally is 0) (HD5192)

Push width: anti-empty bag width (generally wider than dial ruler) (HD5194)

Empty stop: anti-empty bag

Agl judge: anti-cutting angle judgement, judge whether the difference between input pulse angle and actual received angle is overlimit(HD5250)

Revise angle: anti-cutting reverse rotation angle, when torque anti-cutting, the reverse rotation angle is set to 90 degree (calculate the PLSR pulse) (HD5264)

Revise time:  anti-cutting reverse time, the time needed for reverse rotation (calculate PLSR frequency) (HD5266)

Wait num: anti-cutting allowance bag numbers, generally set to 1 (HD5268)

Torque defender: torque anti-cutting

Mark length: anti-cutting photoelectricity cutter distance, the distance between anti-cutting photoelectricity and cutter center (HD5184)

Check width: anti-cutting checking width, if checked the material in the range of left and right postion around cut point, it is considered it will cut the material (HD5186)

Mark defender: photoelectricity anti-cutting